2026 CCDC

Team 02

Palo Alto Firewall

Step-by-Step Configuration Guide

| Network Element | Details |
| --- | --- |
| Firewall Version | PAN-OS 11.0.2 |
| Firewall 1 (FW1) | Outside: 172.16.101.254/24 ¦ Inside: 172.20.242.254/24 ¦ Mgmt: 172.20.242.150 |
| Firewall 2 (FW2) | Outside: 172.16.102.254/24 ¦ Inside: 172.20.240.254/24 ¦ Mgmt: 172.20.240.200 |
| VyOS Router | External: 172.31.21.2/29 ¦ Net1: 172.16.101.1/24 ¦ Net2: 172.16.102.1/24 |
| FW Default Creds | admin / Changeme123 (CHANGE IMMEDIATELY — see Section 1) |

IMPORTANT: Section 1 covers changing ALL default passwords across every device before any other configuration. Do not skip it.

# 1. Change All Default Passwords  (DO THIS FIRST)

IMPORTANT: In CCDC, red team will attempt to take control of your machines within minutes using default credentials. Change every password before configuring anything else.

## 1.1 Palo Alto Firewalls (FW1 and FW2)

Repeat the following for BOTH firewalls. FW1 GUI: https://172.20.242.150  |  FW2 GUI: https://172.20.240.200

WEB GUI METHOD

- Log in with admin / Changeme123

- Navigate to: Device > Administrators

- Click on the admin account in the list

- In the dialog box, fill in:

  - Password: <your new strong password>

  - Confirm Password: <same password>

- Click OK

- Click Commit (top-right) — the password change is NOT live until committed

- You will be logged out. Log back in with your new password to verify it works.

CLI / SSH METHOD

SSH into the firewall management interface or use the console:

ssh admin@172.20.242.150    # FW1

ssh admin@172.20.240.200    # FW2

At the PAN-OS CLI prompt:

# Enter configuration mode

configure

# Set new password — you will be prompted to type and confirm it

set mgt-config users admin password

# Commit the change

commit

TIP: PAN-OS CLI "configure" mode is separate from operational mode. Type "exit" to leave configure mode back to the operational prompt.

## 1.2 VyOS Router

SSH or console into VyOS (172.31.21.2). Default: vyos / changeme

WEB GUI METHOD

VyOS does not have a web GUI. Use CLI only (see below).

CLI / SSH METHOD

ssh vyos@172.31.21.2

# Enter configuration mode

configure

# Replace <newpassword> with your chosen password

set system login user vyos authentication plaintext-password <newpassword>

commit

save

exit

TIP: VyOS hashes the password automatically after "commit". The config file stores it in encrypted form — never in plaintext.

## 1.3 Windows Servers (AD/DNS, Web Server 2019, FTP Server 2022, Win 11 Wks)

WEB GUI METHOD

- Press Ctrl+Alt+Delete (or Ctrl+Alt+End in RDP)

- Click "Change a password"

- Enter the old password, then the new password twice

- Press Enter or click the arrow

For other user accounts on the same machine (e.g., UserOne on Win 11 Wks):

- Open Computer Management: right-click Start > Computer Management

- Expand Local Users and Groups > Users

- Right-click the account > Set Password

- Click Proceed, enter and confirm new password, click OK

CLI / SSH METHOD

Run in an elevated Command Prompt or PowerShell:

REM Change Administrator password

net user administrator <newpassword>

REM Change UserOne password (Win 11 Wks)

net user UserOne <newpassword>

REM Verify account status

net user administrator

Or via PowerShell (more flexible):

Set-LocalUser -Name 'administrator' -Password (ConvertTo-SecureString '<newpw>' -AsPlainText -Force)

## 1.4 Ubuntu & Linux Machines (Ecom, Splunk, Ubuntu Wks)

Default user: sysadmin / changeme (Splunk also has root:changemenow and admin:changeme)

WEB GUI METHOD

Ubuntu Desktop (Ubuntu Wks): Click top-right menu > Settings > Users > click your user > Change Password.

For server machines with no GUI, use CLI only.

CLI / SSH METHOD

# Change sysadmin password (Ubuntu Ecom, Ubuntu Wks)

passwd sysadmin

# Change root password (Splunk / Oracle Linux)

sudo passwd root

# Change Splunk admin account password

sudo /opt/splunk/bin/splunk edit user admin -password <newpassword> -auth admin:changemenow

## 1.5 Fedora Webmail

Default user: sysadmin / changeme

CLI / SSH METHOD

passwd sysadmin

# If root access is needed

sudo passwd root

## 1.6 Password Change Checklist

| Machine | Account(s) | Default Password | Changed? |
| --- | --- | --- | --- |
| FW1 Palo Alto | admin | Changeme123 | [ ] |
| FW2 Palo Alto | admin | Changeme123 | [ ] |
| VyOS Router | vyos | changeme | [ ] |
| Ubuntu Ecom | sysadmin | changeme | [ ] |
| Fedora Webmail | sysadmin | changeme | [ ] |
| Splunk (Oracle Linux) | root / admin | changemenow / changeme | [ ] |
| Ubuntu Wks | sysadmin | changeme | [ ] |
| AD/DNS Server 2019 | administrator | !Password123 | [ ] |
| Web Server 2019 | administrator | !Password123 | [ ] |
| FTP Server 2022 | administrator | !Password123 | [ ] |
| Win 11 Wks | administrator / UserOne | !Password123 / ChangeMe123 | [ ] |

# 2. Network Overview & Architecture

This guide configures both Palo Alto firewalls (PA 11.0.2) for the 2026 CCDC Team 02 network. The topology splits machines across two independent firewall zones, both connecting through a central VyOS router that provides internet access and cross-zone routing.

## 2.1 IP Address Reference — Team 02

| VM | IP Address | OS/Version | Subnet | Public IP |
| --- | --- | --- | --- | --- |
| Ubuntu Ecom | 172.20.242.30 | Ubuntu 24.04.3 | FW1 TRUST | 172.25.22.11 |
| Fedora Webmail | 172.20.242.40 | Fedora 42 | FW1 TRUST | 172.25.22.39 |
| Splunk | 172.20.242.20 | Oracle Linux 9.2 | FW1 TRUST | 172.25.22.9 |
| Ubuntu Wks | DHCP | Ubuntu 24.04.3 | FW1 TRUST | Dynamic |
| AD/DNS Server | 172.20.240.102 | Server 2019 | FW2 TRUST | 172.25.22.155 |
| Web Server | 172.20.240.101 | Server 2019 | FW2 TRUST | 172.25.22.140 |
| FTP Server | 172.20.240.104 | Server 2022 | FW2 TRUST | 172.25.22.162 |
| Win 11 Wks | 172.20.240.100 | Windows 11 24H2 | FW2 TRUST | 172.25.22.144 |
| FW1 Outside | 172.16.101.254/24 | PA 11.0.2 | UNTRUST | — |
| FW1 Inside | 172.20.242.254/24 | PA 11.0.2 | TRUST | — |
| FW2 Outside | 172.16.102.254/24 | PA 11.0.2 | UNTRUST | — |
| FW2 Inside | 172.20.240.254/24 | PA 11.0.2 | TRUST | — |
| VyOS Net1 | 172.16.101.1/24 | VyOS 1.4.3 | FW1 next-hop | — |
| VyOS Net2 | 172.16.102.1/24 | VyOS 1.4.3 | FW2 next-hop | — |

## 2.2 Zone Design

Each Palo Alto firewall uses three zones. Zones must be created before interfaces can be assigned to them.

| Zone Name | Type | Interface | Purpose |
| --- | --- | --- | --- |
| UNTRUST | Layer3 | ethernet1/1 | Faces VyOS router — untrusted/internet side |
| TRUST | Layer3 | ethernet1/2 | Faces internal machines — protected side |

NOTE: Zones are firewall-local objects. You must create them on each firewall separately. FW1 and FW2 each need their own TRUST and UNTRUST zones.

# 3. Firewall 1 Configuration (FW1)

FW1 protects: Ubuntu Ecom (242.30), Fedora Webmail (242.40), Splunk (242.20), Ubuntu Wks (DHCP)

Management GUI: https://172.20.242.150   |   Credentials: admin / Changeme123  (already changed in Section 1)

| 1 | Access the FW1 Web GUI |
| --- | --- |

From the Ubuntu Workstation (behind FW1) or any machine on the management network:

https://172.20.242.150

Accept the self-signed TLS certificate warning, then log in.

TIP: Chrome: click Advanced > Proceed to 172.20.242.150. Firefox: click Accept the Risk and Continue.

| 2 | Create Security Zones (UNTRUST and TRUST) |
| --- | --- |

Zones must exist before you can assign interfaces to them. You will create two zones on FW1.

WEB GUI METHOD

### Create the UNTRUST Zone

- Navigate to: Network > Zones

- Click Add at the bottom of the zone list

- Fill in the zone dialog:

| Field | Value |
| --- | --- |
| Name | UNTRUST |
| Type | Layer3 |
| Zone Protection Profile | (leave blank for now) |
| Log Setting | (leave blank for now) |
| Enable User Identification | Unchecked |

- Click OK

### Create the TRUST Zone

- Click Add again

- Fill in:

| Field | Value |
| --- | --- |
| Name | TRUST |
| Type | Layer3 |
| Zone Protection Profile | (leave blank for now) |
| Enable User Identification | Unchecked |

- Click OK

- You should now see both UNTRUST and TRUST listed under Network > Zones

CLI / SSH METHOD

SSH into FW1 and run:

ssh admin@172.20.242.150

configure

# Create UNTRUST zone

set zone UNTRUST network layer3

# Create TRUST zone

set zone TRUST network layer3

commit

TIP: To verify zones were created: run "show zone" at the operational prompt (outside configure mode). You should see UNTRUST and TRUST listed.

| 3 | Configure Ethernet Interfaces |
| --- | --- |

Now that zones exist, assign each interface to its zone and configure its IP address.

WEB GUI METHOD

### ethernet1/1  —  Outside (UNTRUST)

- Navigate to: Network > Interfaces > Ethernet

- Click on ethernet1/1 in the list

- Interface Type: Layer3

- Click the Config tab:

  - Security Zone: UNTRUST  (select from dropdown)

  - Virtual Router: default

- Click the IPv4 tab:

  - Click Add

  - Type: 172.16.101.254/24

- Click OK

### ethernet1/2  —  Inside (TRUST)

- Click on ethernet1/2 in the list

- Interface Type: Layer3

- Click the Config tab:

  - Security Zone: TRUST

  - Virtual Router: default

- Click the IPv4 tab:

  - Click Add

  - Type: 172.20.242.254/24

- Click OK

CLI / SSH METHOD

configure

# ethernet1/1 — Outside

set network interface ethernet ethernet1/1 layer3 ip 172.16.101.254/24

set network interface ethernet ethernet1/1 layer3 interface-management-profile ''

set zone UNTRUST network layer3 member ethernet1/1

# ethernet1/2 — Inside

set network interface ethernet ethernet1/2 layer3 ip 172.20.242.254/24

set zone TRUST network layer3 member ethernet1/2

commit

| 4 | Configure Virtual Router & Static Routes |
| --- | --- |

WEB GUI METHOD

- Navigate to: Network > Virtual Routers

- Click on default (the pre-existing virtual router)

- Click the Static Routes tab

- Click Add — configure the default route to VyOS:

| Field | Value |
| --- | --- |
| Name | default-route |
| Destination | 0.0.0.0/0 |
| Interface | ethernet1/1 |
| Next Hop Type | IP Address |
| Next Hop IP | 172.16.101.1 (VyOS Net1 interface) |

- Click Add again — route to FW2 internal network:

| Field | Value |
| --- | --- |
| Name | fw2-internal |
| Destination | 172.20.240.0/24 |
| Interface | ethernet1/1 |
| Next Hop Type | IP Address |
| Next Hop IP | 172.16.101.1 (VyOS Router) |

- Click OK to save the virtual router

CLI / SSH METHOD

configure

set network virtual-router default routing-table ip static-route default-route destination 0.0.0.0/0 interface ethernet1/1 nexthop ip-address 172.16.101.1

set network virtual-router default routing-table ip static-route fw2-internal destination 172.20.240.0/24 interface ethernet1/1 nexthop ip-address 172.16.101.1

commit

| 5 | Configure NAT Policy (Outbound Source NAT) |
| --- | --- |

WEB GUI METHOD

- Navigate to: Policies > NAT

- Click Add

- General tab — Name: inside-to-internet

- Original Packet tab:

  - Source Zone: TRUST

  - Destination Zone: UNTRUST

  - Destination Interface: ethernet1/1

  - Source Address: 172.20.242.0/24

  - Destination Address: any

- Translated Packet tab:

  - Translation Type: Dynamic IP And Port

  - Address Type: Interface Address

  - Interface: ethernet1/1

  - IP Address: 172.16.101.254  (auto-selected)

- Click OK

CLI / SSH METHOD

configure

set rulebase nat rules inside-to-internet source-translation dynamic-ip-and-port interface-address interface ethernet1/1

set rulebase nat rules inside-to-internet from TRUST

set rulebase nat rules inside-to-internet to UNTRUST

set rulebase nat rules inside-to-internet source 172.20.242.0/24

set rulebase nat rules inside-to-internet destination any

set rulebase nat rules inside-to-internet service any

commit

| 6 | Create Service Objects & Service Group |
| --- | --- |

These objects define the allowed inbound ports and are referenced in the security policy.

WEB GUI METHOD

### 6a. Create Individual Service Objects

- Navigate to: Objects > Services

- Click Add for each service below:

| Service Name | Protocol | Port | Tab | Purpose |
| --- | --- | --- | --- | --- |
| svc-http | TCP | 80 | Destination | HTTP web traffic |
| svc-https | TCP | 443 | Destination | HTTPS secure web |
| svc-smtp | TCP | 25 | Destination | Email delivery (SMTP) |
| svc-dns-tcp | TCP | 53 | Destination | DNS over TCP |
| svc-dns-udp | UDP | 53 | Destination | DNS over UDP |
| svc-pop3 | TCP | 110 | Destination | Email retrieval (POP3) |
| svc-ftp-cmd | TCP | 21 | Destination | FTP command channel |
| svc-ftp-data | TCP | 20 | Destination | FTP data channel |
| svc-tftp | UDP | 69 | Destination | TFTP file transfer |
| svc-ntp | UDP | 123 | Destination | NTP time sync |

For each row: click Add, enter the Name, set Protocol and Destination Port under the "Destination Port" tab, click OK.

### 6b. Create the Service Group

- Navigate to: Objects > Service Groups

- Click Add

- Name: svc-grp-allowed-inbound

- Click Add under the Members list and add ALL 11 services created above

- Click OK

CLI / SSH METHOD

# Create all service objects

set application-group ''

set service svc-http protocol tcp port 80

set service svc-https protocol tcp port 443

set service svc-smtp protocol tcp port 25

set service svc-dns-tcp protocol tcp port 53

set service svc-dns-udp protocol udp port 53

set service svc-pop3 protocol tcp port 110

set service svc-ftp-cmd protocol tcp port 21

set service svc-ftp-data protocol tcp port 20

set service svc-tftp protocol udp port 69

set service svc-ntp protocol udp port 123

# Create the service group

set service-group svc-grp-allowed-inbound members svc-http

set service-group svc-grp-allowed-inbound members svc-https

set service-group svc-grp-allowed-inbound members svc-smtp

set service-group svc-grp-allowed-inbound members svc-dns-tcp

set service-group svc-grp-allowed-inbound members svc-dns-udp

set service-group svc-grp-allowed-inbound members svc-pop3

set service-group svc-grp-allowed-inbound members svc-ftp-cmd

set service-group svc-grp-allowed-inbound members svc-ftp-data

set service-group svc-grp-allowed-inbound members svc-tftp

set service-group svc-grp-allowed-inbound members svc-ntp

commit

| 7 | Configure Security Policies |
| --- | --- |

IMPORTANT: Rules are matched TOP-DOWN — the first match wins. Create them in the exact order shown. Rules 1-4 must appear above Rule 5.

WEB GUI METHOD

Navigate to: Policies > Security. Delete any existing default rules, then Add each rule below:

### Rule 1 — Allow TRUST to TRUST (intra-zone / machine-to-machine)

| Field | Value |
| --- | --- |
| Name | allow-trust-to-trust |
| Source Zone | TRUST |
| Destination Zone | TRUST |
| Source Address | any |
| Destination Address | any |
| Application | any |
| Service | any |
| Action | Allow |

### Rule 2 — Allow TRUST to UNTRUST (outbound internet)

| Field | Value |
| --- | --- |
| Name | allow-trust-to-untrust |
| Source Zone | TRUST |
| Destination Zone | UNTRUST |
| Source Address | 172.20.242.0/24 |
| Destination Address | any |
| Application | any |
| Service | any |
| Action | Allow |

### Rule 3 — Allow TRUST to FW2 Network (cross-firewall via VyOS)

| Field | Value |
| --- | --- |
| Name | allow-to-fw2-network |
| Source Zone | TRUST |
| Destination Zone | UNTRUST |
| Source Address | 172.20.242.0/24 |
| Destination Address | 172.20.240.0/24 |
| Application | any |
| Service | any |
| Action | Allow |

### Rule 4 — Allow Inbound Permitted Services (UNTRUST to TRUST)

| Field | Value |
| --- | --- |
| Name | allow-inbound-services |
| Source Zone | UNTRUST |
| Destination Zone | TRUST |
| Source Address | any |
| Destination Address | 172.20.242.0/24 |
| Application | any |
| Service | svc-grp-allowed-inbound (the group from Step 6) |
| Action | Allow |

### Rule 5 — Deny All Other Inbound (Default Deny)

| Field | Value |
| --- | --- |
| Name | deny-all-inbound |
| Source Zone | UNTRUST |
| Destination Zone | TRUST |
| Source Address | any |
| Destination Address | any |
| Application | any |
| Service | any |
| Action | Deny |

CLI / SSH METHOD

configure

# Rule 1 — trust to trust

set rulebase security rules allow-trust-to-trust from TRUST

set rulebase security rules allow-trust-to-trust to TRUST

set rulebase security rules allow-trust-to-trust source any

set rulebase security rules allow-trust-to-trust destination any

set rulebase security rules allow-trust-to-trust application any

set rulebase security rules allow-trust-to-trust service any

set rulebase security rules allow-trust-to-trust action allow

# Rule 2 — outbound internet

set rulebase security rules allow-trust-to-untrust from TRUST

set rulebase security rules allow-trust-to-untrust to UNTRUST

set rulebase security rules allow-trust-to-untrust source 172.20.242.0/24

set rulebase security rules allow-trust-to-untrust destination any

set rulebase security rules allow-trust-to-untrust application any

set rulebase security rules allow-trust-to-untrust service any

set rulebase security rules allow-trust-to-untrust action allow

# Rule 3 — cross-firewall to FW2

set rulebase security rules allow-to-fw2-network from TRUST

set rulebase security rules allow-to-fw2-network to UNTRUST

set rulebase security rules allow-to-fw2-network source 172.20.242.0/24

set rulebase security rules allow-to-fw2-network destination 172.20.240.0/24

set rulebase security rules allow-to-fw2-network application any

set rulebase security rules allow-to-fw2-network service any

set rulebase security rules allow-to-fw2-network action allow

# Rule 4 — allowed inbound ports

set rulebase security rules allow-inbound-services from UNTRUST

set rulebase security rules allow-inbound-services to TRUST

set rulebase security rules allow-inbound-services source any

set rulebase security rules allow-inbound-services destination 172.20.242.0/24

set rulebase security rules allow-inbound-services application any

set rulebase security rules allow-inbound-services service svc-grp-allowed-inbound

set rulebase security rules allow-inbound-services action allow

# Rule 5 — deny everything else inbound

set rulebase security rules deny-all-inbound from UNTRUST

set rulebase security rules deny-all-inbound to TRUST

set rulebase security rules deny-all-inbound source any

set rulebase security rules deny-all-inbound destination any

set rulebase security rules deny-all-inbound application any

set rulebase security rules deny-all-inbound service any

set rulebase security rules deny-all-inbound action deny

commit

| 8 | Commit FW1 Configuration |
| --- | --- |

GUI: click the Commit button (top-right corner, pencil/checkmark icon). Wait for "Commit Succeeded".

WARNING: Nothing goes live until you Commit. If a mistake is made, use Device > Setup > Operations > Revert to Last Saved Configuration before committing.

# 4. Firewall 2 Configuration (FW2)

FW2 protects: AD/DNS Server (240.102), Web Server (240.101), FTP Server (240.104), Win 11 Wks (240.100)

Management GUI: https://172.20.240.200   |   All steps mirror FW1 with FW2-specific IPs.

NOTE: Every step in this section is identical in structure to Section 3. Only IP addresses differ. Work through Steps 1-8 in the same order.

| 1 | Access FW2 Web GUI |
| --- | --- |

https://172.20.240.200

Log in with admin and your new password (changed in Section 1).

| 2 | Create Security Zones on FW2 |
| --- | --- |

Zones must be created fresh on FW2 — they do not carry over from FW1.

WEB GUI METHOD

- Navigate to: Network > Zones > Add

- Create UNTRUST zone:

  - Name: UNTRUST

  - Type: Layer3

  - Click OK

- Click Add again and create TRUST zone:

  - Name: TRUST

  - Type: Layer3

  - Click OK

CLI / SSH METHOD

ssh admin@172.20.240.200

configure

set zone UNTRUST network layer3

set zone TRUST network layer3

commit

| 3 | Configure Ethernet Interfaces on FW2 |
| --- | --- |

WEB GUI METHOD

### ethernet1/1 — Outside (UNTRUST) on FW2

| Field | Value |
| --- | --- |
| Interface Type | Layer3 |
| Security Zone | UNTRUST |
| IPv4 Address | 172.16.102.254/24 |

### ethernet1/2 — Inside (TRUST) on FW2

| Field | Value |
| --- | --- |
| Interface Type | Layer3 |
| Security Zone | TRUST |
| IPv4 Address | 172.20.240.254/24 |

CLI / SSH METHOD

configure

set network interface ethernet ethernet1/1 layer3 ip 172.16.102.254/24

set zone UNTRUST network layer3 member ethernet1/1

set network interface ethernet ethernet1/2 layer3 ip 172.20.240.254/24

set zone TRUST network layer3 member ethernet1/2

commit

| 4 | Static Routes on FW2 |
| --- | --- |

WEB GUI METHOD

Navigate to: Network > Virtual Routers > default > Static Routes. Add two routes:

| Name | Destination | Interface | Next Hop IP |
| --- | --- | --- | --- |
| default-route | 0.0.0.0/0 | ethernet1/1 | 172.16.102.1 (VyOS Net2) |
| fw1-internal | 172.20.242.0/24 | ethernet1/1 | 172.16.102.1 (VyOS Router) |

CLI / SSH METHOD

configure

set network virtual-router default routing-table ip static-route default-route destination 0.0.0.0/0 interface ethernet1/1 nexthop ip-address 172.16.102.1

set network virtual-router default routing-table ip static-route fw1-internal destination 172.20.242.0/24 interface ethernet1/1 nexthop ip-address 172.16.102.1

commit

| 5 | NAT Policy on FW2 |
| --- | --- |

WEB GUI METHOD

Navigate to: Policies > NAT > Add:

| Field | Value |
| --- | --- |
| Name | inside-to-internet |
| Source Zone | TRUST |
| Destination Zone | UNTRUST |
| Source Address | 172.20.240.0/24 |
| Translation Type | Dynamic IP And Port — Interface Address — ethernet1/1 |

CLI / SSH METHOD

configure

set rulebase nat rules inside-to-internet source-translation dynamic-ip-and-port interface-address interface ethernet1/1

set rulebase nat rules inside-to-internet from TRUST

set rulebase nat rules inside-to-internet to UNTRUST

set rulebase nat rules inside-to-internet source 172.20.240.0/24

set rulebase nat rules inside-to-internet destination any

set rulebase nat rules inside-to-internet service any

commit

| 6 | Create Service Objects & Group on FW2 |
| --- | --- |

Repeat Step 6 from Section 3 exactly — create the same 11 service objects and the svc-grp-allowed-inbound group. Objects are firewall-local and must be created on each device.

| 7 | Security Policies on FW2 |
| --- | --- |

Create the same 5 rules as FW1, substituting 172.20.240.0/24 where 172.20.242.0/24 appeared:

| Rule Name | Src Zone | Dst Zone | Src Address | Service | Action |
| --- | --- | --- | --- | --- | --- |
| allow-trust-to-trust | TRUST | TRUST | any | any | Allow |
| allow-trust-to-untrust | TRUST | UNTRUST | 172.20.240.0/24 | any | Allow |
| allow-to-fw1-network | TRUST | UNTRUST | 172.20.240.0/24 dst: .242.0/24 | any | Allow |
| allow-inbound-services | UNTRUST | TRUST | any dst: 172.20.240.0/24 | svc-grp | Allow |
| deny-all-inbound | UNTRUST | TRUST | any | any | Deny |

| 8 | Commit FW2 Configuration |
| --- | --- |

Click Commit (top-right). Wait for "Commit Succeeded" before proceeding.

# 5. VyOS Router Configuration

VyOS is the central hub connecting both firewall networks and providing internet access. It has no GUI — CLI only.

| Interface | IP Address | Connected To |
| --- | --- | --- |
| eth0 (external) | 172.31.21.2/29 | Upstream internet gateway |
| eth1 (net1) | 172.16.101.1/24 | Firewall 1 outside (172.16.101.254) |
| eth2 (net2) | 172.16.102.1/24 | Firewall 2 outside (172.16.102.254) |

| 1 | SSH into VyOS |
| --- | --- |

ssh vyos@172.31.21.2

Password: the new password set in Section 1.2

configure

| 2 | Verify / Set Interface Addresses |
| --- | --- |

set interfaces ethernet eth0 address '172.31.21.2/29'

set interfaces ethernet eth1 address '172.16.101.1/24'

set interfaces ethernet eth2 address '172.16.102.1/24'

TIP: If these are already set, VyOS will respond with "Value already exists" — that is fine, just continue.

| 3 | Set Default Route to Internet |
| --- | --- |

set protocols static route 0.0.0.0/0 next-hop 172.31.21.1

Replace 172.31.21.1 with the actual upstream gateway IP if different.

| 4 | Configure NAT Masquerade for Both FW Networks |
| --- | --- |

set nat source rule 10 outbound-interface eth0

set nat source rule 10 source address '172.16.101.0/24'

set nat source rule 10 translation address masquerade

set nat source rule 20 outbound-interface eth0

set nat source rule 20 source address '172.16.102.0/24'

set nat source rule 20 translation address masquerade

TIP: To verify existing NAT rules: exit configure mode and run "show nat source rules". Rules 10 and 20 should appear.

| 5 | Commit and Save |
| --- | --- |

commit

save

exit

# 6. Verification & Testing

| 1 | Verify Intra-Zone Connectivity (FW1 machines talking to each other) |
| --- | --- |

From Ubuntu Ecom (172.20.242.30):

ping 172.20.242.40     # Fedora Webmail

ping 172.20.242.20     # Splunk

All should succeed. If not: check the allow-trust-to-trust rule on FW1 and verify both machines have 172.20.242.254 as their default gateway.

| 2 | Verify Cross-Firewall Connectivity (FW1 to FW2 machines) |
| --- | --- |

From Ubuntu Ecom (172.20.242.30):

ping 172.20.240.102    # AD/DNS Server

ping 172.20.240.101    # Web Server

ping 172.20.240.104    # FTP Server

Troubleshoot if failing: (1) FW1 static route to 172.20.240.0/24 exists, (2) FW2 static route to 172.20.242.0/24 exists, (3) VyOS routes between eth1 and eth2.

| 3 | Verify Internet Access |
| --- | --- |

From any internal machine on either firewall:

ping 8.8.8.8

curl http://example.com

If failing: verify NAT rules on the respective firewall and VyOS masquerade rules.

| 4 | Verify Allowed Inbound Ports |
| --- | --- |

| Service | Port | Test Command | Expected Result |
| --- | --- | --- | --- |
| HTTP | 80/TCP | curl http://<IP> | 200 OK response |
| HTTPS | 443/TCP | curl -k https://<IP> | 200 OK or cert page |
| SMTP | 25/TCP | telnet <IP> 25 | 220 SMTP banner |
| DNS | 53/UDP | nslookup test.com <IP> | DNS response received |
| POP3 | 110/TCP | telnet <IP> 110 | +OK POP3 greeting |
| FTP | 21/TCP | ftp <IP> | 220 FTP server ready |
| TFTP | 69/UDP | tftp <IP> | TFTP connection opens |
| NTP | 123/UDP | ntpdate -q <IP> | Time response received |

| 5 | Verify Blocked Traffic |
| --- | --- |

From outside, try a port NOT in the allowed list:

nc -zv <public_IP> 22     # SSH — should timeout or be refused

nc -zv <public_IP> 3389   # RDP — should timeout or be refused

If either connects, review the deny-all-inbound rule and confirm it is below the allowed-services rule.

| 6 | Use Traffic Logs to Troubleshoot |
| --- | --- |

- Navigate to: Monitor > Logs > Traffic

- Use the filter bar: enter (addr.src in 172.20.242.0/24) or similar

- Look for Action = deny to spot blocked flows

- The "Rule" column shows which policy matched

- Green = allowed, Red = denied

TIP: If a CCDC scoring check fails, go here first. You can see exactly which packets arrived, which rule matched, and why they were dropped — all in real time.

# 7. Quick Reference Card

## Common Troubleshooting

| Problem | Check / Fix |
| --- | --- |
| Machine can't ping its gateway | Verify interface IP and zone on the firewall. Check machine default gateway setting. |
| No internet from internal | Check NAT rule + allow-trust-to-untrust rule. Check VyOS masquerade (show nat source rules). |
| Can't reach FW2 from FW1 | Check static routes on both FWs. Check VyOS eth1/eth2 routing. Check allow-to-fw2 rule. |
| Inbound service not scoring | Monitor > Traffic Logs. Check service is in svc-grp-allowed-inbound. Check rule order. |
| Changes not taking effect | You must Commit. Look for pending changes number in top-right of GUI. |
| Zone assignment missing | Network > Interfaces — confirm each interface shows the correct zone in the Zone column. |
| Locked out of firewall | Console: boot to maintenance mode. Select "Reset admin password" from the menu. |

## PAN-OS Operational CLI Quick Commands

These run in operational mode (NOT in configure mode — type "exit" first if in configure):

| Command | Purpose |
| --- | --- |
| show interface all | List all interfaces and their status/IPs |
| show zone | List all zones and their member interfaces |
| show routing route | Show active routing table |
| show running nat-policy | Show active NAT rules |
| show running security-policy | Show active security policy rules |
| ping host 8.8.8.8 | Ping from the firewall itself |
| show system info | Show hostname, model, PAN-OS version |

END OF DOCUMENT  —  Team 02  —  2026 CCDC